| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 10 | Introduction |
| Lesson 3 |  | Rules of Conflict Notification |  |  |  |

Topics Covered: - General rules for conflict notification - Rules of sector control during conflict notification (CU of a point of violation)

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| In this lesson, we’ll explain the rules for conflict notification, and the rules of sector control during conflict notification. | CU of point of violation. Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 20 | Introduction |
| Lesson 3 |  | Conflict Notification |  |  |  |

Conflict Detection and Notification: - Current plans checked for conflicts automatically - Automation applies reasoning to determine when to notify about a conflict. - Prevents false alerts Automated Problem Detection (APD) (ACL with alerts) Detects

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	    Production Notes:

| A current plan is checked for conflicts automatically whenever there is reconformance or a flight plan amendment. | Graphic and bullet 1. |
| --- | --- |
| Although Automated Problem Detection, or APD, may detect a conflict, you may not be notified immediately. |  |
| The automation applies reasoning to determine when to notify about a conflict. | Add bullet 2 |
| For example, if the automation detects a high probability conflict eighteen minutes into the future, it may notify you immediately. |  |
| Conversely, if a low probability conflict is detected, the automation may wait until ten minutes before the conflict to notify you. |  |
| This is done to prevent false alerts - when low probability conflicts become no conflict. If the low probability conflict becomes a high probability conflict, the automation adjusts the notification time accordingly. | Add bullet 3 |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 30 | Rules |
| Lesson 3 |  | Conflict Notification |  |  |  |

Rules of Conflict Notification: - Only one sector gets the alert at a time - Notification is generally provided to sector where violation is first predicted to occur for aircraft-to-aircraft problems - Notification of alerts will be transferred to the next controlling sector upon handoff Automated Problem Detection (APD) (ACL with alerts) Detects Rule #1 Rule #2 Rule #3 Rule #4 Rule #5 Rule #6

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

| Only one sector gets the alert at a time. This prevents confusion that could be caused by notifying a number of sectors of the same conflict. | Circle graphic and ACL, no Rules yet. Add bullet 1. |
| --- | --- |
| When the decision is made by the system to notify the sector position, the notification is generally provided to the sector where the violation is first predicted to occur for aircraft-to-aircraft problems. | Add bullet 2. |
| Notification of alerts will be transferred to the next controlling sector upon handoff. | Add bullet 3. |
| To better understand how the system determines which D-position is assigned a current plan alert, click on each of the rules used in the determination, starting with Rule number one, in descending order. The system applies the rules in order, from one to six, with five being a catch-all. | Add Rules buttons. Only Rule #1 is active. The rest are grayed out. Activate them one at a time as the previous rule is viewed. Add green checks next to the rule button after each is viewed. |

Branching:

| Next | 100 | Back | 20 | Rule 1 | 40 |
| --- | --- | --- | --- | --- | --- |
| Rule 2 | 50 | Rule 3 | 60 | Rule 4 | 70 |
| Rule 5 | 80 | Rule 6 | 90 |  |  |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 40 | Rule #1 |
| Lesson 3 |  | Conflict Notification |  |  |  |

Initial Point of Violation Rule #1: At the predicted start time of the conflict, if both aircraft’s points of violation are in the same sector, that sector will receive the conflict notification. Return Click the Return Button Example

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

| To hear an example, click the button. |  |
| --- | --- |
| At the predicted start time of the conflict, | Graphic depicting multiple sectors. Use URET M5L1, page 24, Rule 1 as guide. No highlight around POV. Don’t play audio until EXAMPLE is selected. |
| Jetlink thirty fifty-three and Piedmont thirty-two thirty-seven’s points of violation are both located within a single sector. | Add yellow square around Point of violation and add line and label. |
| Therefore, sector E will receive the conflict notification. | Add highlight around Sector E. Add Return button and prompt when audio is done. |

Branching:

| Next | inactive | Back | inactive | Return | 30 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 50 | Rule #2 |
| Lesson 3 |  | Conflict Notification |  |  |  |

Initial Point of Violation Rule #2: At the predicted start time of the conflict, if the points of violation are in different sectors, and only one of those sectors controls an aircraft involved in the conflict, the sector containing the point of violation and currently controlling one of the aircraft receives the conflict notification. Return Click the Return Button Example

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

| At the predicted start time of the conflict, | Graphic depicting multiple sectors. Use URET M5L1, page 24, Rule 2 as guide. No highlight around POV. Don’t play audio until EXAMPLE is selected. |
| --- | --- |
| Jetlink thirty fifty-three and US Air fifteen sixty-nine’s points of violation are located within different sectors – sectors C and D. | Add yellow square around Point of violation and add line and label. |
| Since Sector D does not have track control of either aircraft and sector C does, |  |
| therefore, sector C receives the conflict notification. | Add highlight around Sector C. Add Return button and prompt when audio is done. |

Branching:

| Next | inactive | Back | inactive | Return | 30 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 60 | Rule #3 |
| Lesson 3 |  | Conflict Notification |  |  |  |

Intersection of Conflict Rule #3: At the predicted start time of the conflict, if each aircraft’s point of violation is in a different sector and the conflict intersection is in only one of the sectors, then the sector controlling the airspace containing the conflict intersection will receive the notification. Return Click the Return Button Example

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

| At the predicted start time of the conflict, | Graphic depicting multiple sectors. Use URET M5L1, page 24, Rule 3 as guide. No highlight around POV. Don’t play audio until EXAMPLE is selected. |
| --- | --- |
| Jetlink thirty fifty-three and Piedmont thirty-two thirty-seven’s points of violation are located within different sectors. | Add yellow square around Point of violation and add line and label. |
| Only one of the sectors contains the intersection of the conflicting trajectories. The position to receive conflict notification is the one controlling the airspace containing the intersection. |  |
| Therefore, sector C will receive the conflict notification. | Add highlight around Sector C. Add Return button and prompt when audio is done. |

Branching:

| Next | inactive | Back | inactive | Return | 30 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 70 | Rule #4 |
| Lesson 3 |  | Conflict Notification |  |  |  |

Rule #4: At the predicted start time of the conflict, if each aircraft’s point of violation is in a different sector and one aircraft is predicted to reach a sector boundary before the other aircraft reaches another sector boundary, then the sector that is predicted to be reached first will receive conflict notification. Return Click the Return Button Initial Point of Violation Example

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

| At the predicted start time of the conflict, | Graphic depicting multiple sectors. Use URET M5L1, page 24, Rule 4 as guide. No highlight around POV. Don’t play audio until EXAMPLE is selected. |
| --- | --- |
| Jetlink thirty fifty-three and Piedmont thirty-two thirty-seven’s points of violation are located within different sectors. If one aircraft is predicted to reach one of these sector’s boundary before the other aircraft reaches the other sector’s boundary, then the sector that is predicted to be reached first will receive conflict notification. As you can see, Piedmont thirty-two thirty-seven will be in sector D’s airspace before Jetlink thirty fifty-three gets to sector C’s airspace. | Add yellow square around Point of violation and add line and label. |
| Therefore, sector D will receive the conflict notification. | Add highlight around Sector D. Add Return button and prompt when audio is done. |

Branching:

| Next | inactive | Back | inactive | Return | 30 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 80 | Rule #5 |
| Lesson 3 |  | Conflict Notification |  |  |  |

Rule #5: If none of the previous rules apply, the system arbitrarily assigns the alert to one of the sectors containing the initial points of violation. Return Click the Return Button Initial Point of Violation

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

|  | Graphic depicting multiple sectors. Use URET M5L1, page 24, Rule 5 as guide. Highlight around POV. |
| --- | --- |

Branching:

| Next | inactive | Back | inactive | Return | 30 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Rules of Notification |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 90 | Rule #6 |
| Lesson 3 |  | Conflict Notification |  |  |  |

Rule #6: Aircraft-to- airspace alerts are notified to the sector that has track control of the aircraft. These alerts are posted to the sector at a facility determined time, up to 40 minutes in advance. Return Click the Return Button Example MOA

Quit Options Next GoTo Back Section 2: Rules of Notification Repeat

- Audio:	    Production Notes:

| At the predicted start time of the conflict, | Graphic depicting multiple sectors. Use URET M5L1, page 24, Rule 4 as guide. No highlight around POV. Don’t play audio until EXAMPLE is selected. |
| --- | --- |
| Jetlink 30-53’s trajectory is predicted to violate the M-O-A in sector E. Since sector A has track control, |  |
| the blue alert is notified to sector A. | Add highlight around Sector A. Add Return button and prompt when audio is done. |

Branching:

| Next | inactive | Back | inactive | Return | 30 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  |  |  |  |
| Lesson 3 |  | Conflict Notification | Frame | 100 |  |

(pic of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 3: Exercises Repeat

- Audio:	    Production Notes:

|  | Photo of URET |
| --- | --- |

Branching:

| Next | 110 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  |  |  |  |
| Lesson 3 |  | Conflict Notification | Frame | 110 | Exercise 1 |

Exercise 1 of 3 What happens to alert notifications at handoff? Click on your choice. (Place Correct and Incorrect text feedback here when appropriate) You must clear all alerts before handing off aircraft. Alert notification passes to the next sector at hand off. URET removes all alerts before aircraft are handed off. Alerts will flash for acknowledgement by the receiving sector at handoff.

Quit Options Next GoTo Back Section 3: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

| Correct! | Correct answer B. |
| --- | --- |
| If select A: | Show Incorrect. Try again. in cyan text at bottom left. |
| You are not required to clear the alerts before you handoff aircraft. Try again. | Incorrect feedback. |
| If select C: | Show Incorrect. Try again. In cyan text at bottom left. |
| URET does not remove alerts until there is no longer a predicted conflict. Try again. |  |
| If select D: | Show Incorrect. Try again. In cyan text at bottom left. |
| Alerts do not flash in URET. Try again. |  |

Branching:

| Next | 120 | Back | 100 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  |  |  |  |
| Lesson 3 |  | Conflict Notification | Frame | 120 | Exercise 2 |

Exercise 2 of 3 Which sector(s) is notified in the event of an alert? Click on your choice. (Place Correct and Incorrect text feedback here when appropriate) Only one sector is notified to prevent confusion. All sectors controlling aircraft involved in the conflict. All sectors controlling aircraft involved in the conflict and sectors predicted to be involved in the future. All sectors where the conflict can be resolved.

Quit Options Next GoTo Back Section 3: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

| Very Good. | Correct answer A. |
| --- | --- |
| If select B: | Show Incorrect. Try again. in cyan text at bottom left. |
| No. It would be confusing if all sectors controlling aircraft involved in the conflict were notified. Try again. | Incorrect feedback. |
| If select C: | Show Incorrect. Try again. In cyan text at bottom left. |
| Incorrect. URET does not predict future sectors which may be involved in the conflict. Try again. |  |
| If select D: | Show Incorrect. Try again. In cyan text at bottom left. |
| No. It would be confusing to notify more than one sector of a conflict. Try again. |  |

Branching:

| Next | 130 | Back | 110 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  |  |  |  |
| Lesson 3 |  | Conflict Notification | Frame | 130 | Exercise 3 |

Exercise 3 of 3 Which sector(s) is notified in the event of an aircraft to airspace alert? Click on your choice. (Place Correct and Incorrect text feedback here when appropriate) The sector that contains the MOA involved in the conflict. Both the sector with track control of the aircraft and the sector containing the MOA involved in the conflict. The sector that has track control of the aircraft at a facility determined time, up to 40 minutes in advance. The sector through which the aircraft passes before being handed off to the sector containing the MOA involved in the conflict.

Quit Options Next GoTo Back Section 3: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

| That’s right. | Correct answer C. |
| --- | --- |
| If select A or D: | Show Incorrect. Try again. in cyan text at bottom left. |
| Not necessarily. The notification is based on the time the aircraft will reach the M-O-A. Try again. | Incorrect feedback. |
| If select B: | Show Incorrect. Try again. In cyan text at bottom left. |
| Incorrect. Only one sector is notified of an alert. Try again. |  |

Branching:

| Next | 140 | Back | 120 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  |  |  |  |
| Lesson 3 |  | Conflict Notification | Frame | 140 | Summary |

## Exercise; Correct 1st Try?. 1. What happens to alert notifications at handoff?. 2. Which sector(s) is notified in the event of an alert?. 3. Which sector(s) is notified in the event of an aircraft to airspace alert?. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 3: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

|  | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 150 | Back | 130 | If click Ex 1 | 30 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 30 | If click Ex 3 | 9 |  |  |

| Course: |  | URET Phase 2 | Section: | 4 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 150 | Summary |
| Lesson 3 |  | Rules of Conflict Notification |  |  |  |

Topics Covered: - General rules for conflict notification - Rules of sector control during conflict notification (CU of a point of violation)

Quit Options Next GoTo Back Section 4: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson explaining the Rules of Conflict Notification. | CU of point of violation. |
| --- | --- |

Branching:

| Next | EXIT | Back | 140 |  |  |
| --- | --- | --- | --- | --- | --- |