| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 10 | Introduction |
| Lesson 2 |  | Wind Grid Display |  |  |  |

Topics Covered: - Purpose of the Wind Grid Display - Switch between temperature and wind data - Recentering the display - Changing the display range - Change display altitude (CU of clouds)

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| In this lesson, we’ll introduce the Wind Grid Display. We’ll discuss the purpose and function of the display, and explain how to: switch between temperature and winds, recenter the display, change the range, and, change the altitude for the display. | CU of clouds. Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Wind and Temperature Data |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 20 | Toggling |
| Lesson 2 |  | Wind Grid Display |  |  |  |

To Toggle the Display: - Left-click on Show Temp/ Remove Temp Try It: Toggle between wind and temperature data. When you’re finished, click NEXT. Wind Grid Show Temp

Quit Options Next GoTo Back Section 2: Wind and Temperature Data Repeat

- Audio:	    Production Notes:

| The wind grid presents the latest wind or temperature data from Warp overlaid on a map of the Center sectors. | Wind grid with altitude set at 30,000 feet. Range at 25. |
| --- | --- |
| The date and time indicator provides the hour and date of the forecast being used. | Highlight date and time. |
| Use the Show Temp button to toggle between wind and temperature data. | Highlight Show Temp button |
| When temperature data is displayed, the button changes to Remove Temp. | Change to Temperature data and highlight Remove Temp button. |
| Try it. Toggle between wind and temperature data. | Add bullet and Try It text. Free practice. |
| If left click on Show Temp/Remove Temp button: | Change button to gray background. |
| Change between Show Temp to Remove Temp with black background. Change the display between temperature and wind data. |  |
| If middle or right click or left click anywhere but Show Temp/Remove TEmp: |  |
| Incorrect. To change the toggle the display, left click on the Show Temp-Remove temp button. Try again. | Incorrect feedback. Highlight Show Temp/Remove Temp button. |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | Recentering |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 30 | Recentering |
| Lesson 2 |  | Wind Grid Display |  |  |  |

To Recenter the Display: - Left-click on Recenter - Left-click on a new geographic location on map Try It: Recenter the map over the Elkins Airport symbol. Recenter PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 3: Recentering Repeat

- Audio:	    Production Notes:

| You can use the Recenter button to change the center point of the map on the wind grid display. | Wind grid with altitude set at 30,000 feet. Range at 25. |
| --- | --- |
| Try it. Recenter the map over the Elkins airport symbol. | Add bullets and Try It text. |
| Step 1: Left-click on Recenter | If done correctly, change cursor to an X. Change button background to gray. |
| If middle or right click or left click anywhere but Recenter: |  |
| No. You need to left-click on the Recenter button. Try again. | Highlight Recenter button. |
| Step 2: Left-click on the circle EKN area. |  |
| Correct. If you try to offset the map outside its defined limits, you’ll hear an alarm tone. | Change circle EKN to the center of the display. Show Correct feedback. |
| If left click anywhere but EKN airport symbol: |  |
| No. For this exercise, please select the Elkins airport symbol as the new map center point. Try again. | Incorrect feedback. Highlight circle EKN. |
| If middle or right click anywhere: |  |
| No, you need to left-click on the new map center point. Try again. | Incorrect feedback. |

Branching:

| Next | 40 | Back | 20 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Range |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 40 | Changing the Range |
| Lesson 2 |  | Wind Grid Display |  |  |  |

To Change the Range: - Left-click on RANGE button to decrease Range - Middle-click on RANGE button to increase Range Try It: Change the Range of the Wind Grid Display. When you’re finished, click NEXT. Range PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 4: Range Repeat

- Audio:	    Production Notes:

| Use the Range button to increase or decrease the wind grid display’s range. | Wind grid with altitude set at 30,000 feet. Range at 25. |
| --- | --- |
| The scale indicator will change to indicate the new range. If you try to adjust the range lower than the lowest setting or higher than the highest setting, an alarm tone will sound. | Highlight scale. |
| Try it. Change the Range of the Wind Grid Display. When you’re finished, click NEXT. | Add bullets and Try It text. Free practice |
| If left or middle-click on Range: |  |
| Change display range and scale. |  |
| If left or middle click anywhere but Range: |  |
| Incorrect. You need to left or middle click on the Range button. Try again. | Incorrect feedback. Highlight Range. |
| If right click anywhere: |  |
| Left-click on Range to decrease the range or middle-click to increase it. Try again. | Incorrect feedback. |

Branching:

| Next | 50 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | Altitude |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 50 | Changing the Altitude |
| Lesson 2 |  | Wind Grid Display |  |  |  |

To Change the Altitude: - Left-click on ALTITUDE button - Left-click on the desired altitude in the Altitude menu or type desired altitude in input box and press ENTER Try It: Change the Wind Grid altitude to 14,000 feet. Altitude PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 5: Altitude Repeat

- Audio:	    Production Notes:

| The winds will be displayed at the default altitude of thirty thousand feet, but you can change the display altitude to any value between zero and fifty-nine thousand feet. | Wind grid with altitude set at 30,000 feet. Range at 25. |
| --- | --- |
| Try it. Change the display altitude to one four thousand. | Add bullets and Try It text. |
| Step 1: If left-click on Altitude: |  |
| Change Altitude button background to gray, bring up Altitude Menu as shown below. |  |
| If middle or right click anywhere or left click anywhere but Altitude: |  |
| Incorrect. You need to left click on the Altitude button. Try again. | Incorrect feedback. Highlight Altitude. |
| Step 2: If left-click on 140 or type 140 and press enter: |  |
| Good. You can leave the altitude menu up, or left click on EXIT to remove it. | Correct feedback. Change display to an altitude of 14,000 feet. Background highlight 140 value in menu and change value in input area to 140.Highlight EXIT. |
| If middle or right click anywhere or left click anywhere but 140: |  |
| No. You need to left-click on one four zero. Try again. | Incorrect feedback. Highlight 140. |
| If type a value other than 140: |  |
| You have not typed the correct value. Type one four zero. Try again. | Incorrect feedback. |

Branching:

| Next | 60 | Back | 40 |  |  |
| --- | --- | --- | --- | --- | --- |

Altitude Menu

0

220

230

590

EXIT

300

| Course: |  | URET Phase 2 | Section: | 6 | Problem Indicators |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 60 | Problem Indicators |
| Lesson 2 |  | Wind Grid Display |  |  |  |

Wind Grid Altitude PERKS EKN NO WEATHER DATA Problem Indicators: - “NO WEATHER DATA” – no weather data is available for display - Stale Weather Indicator - Time is yellow to indicate that data does not meet criteria for current data

Quit Options Next GoTo Back Section 6: Problem Indicators Repeat

- Audio:	    Production Notes:

| There are two error messages associate with the Wind Grid. | Wind grid with altitude set at 30,000 feet. Range at 25. |
| --- | --- |
| “No Weather Data” will appear in large yellow letters in the center of the display when no weather data is available. | Add “NO WEATHER DATA” in yellow text in center of display. |
| And the time indicator will be yellow when weather data available for display does meet the criteria for current data. | Change time to yellow and highlight. |

Branching:

| Next | 70 | Back | 50 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 70 | Introduction |
| Lesson 2 |  | Wind Grid Display |  |  |  |

(photo of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 7: Exercises Repeat

- Audio:	    Production Notes:

|  | Photo of URET |
| --- | --- |

Branching:

| Next | 80 | Back | 60 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 80 | Exercise 1 |
| Lesson 2 |  | Wind Grid Display |  |  |  |

Exercise 1 of 4 Display winds on the Wind Grid. (Place Correct and Incorrect text feedback here when appropriate) Remove Temp PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 7: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Wind grid with altitude set at 30,000 feet. Range at 25. |
| --- | --- |
| If left click on Remove Temp button: | Change button to gray background. |
| That’s right. | Change Remove Temp to Show Temp with black background. Highlight. Change the display to wind data. Show Correct feedback. |
| If middle or right click or left click anywhere but Remove Temp: |  |
| Incorrect. To change the display to wind, left click on Remove Temp. Try again. | Incorrect feedback. Highlight Remove Temp button. |

Branching:

| Next | 90 | Back | 70 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 90 | Exercise 2 |
| Lesson 2 |  | Wind Grid Display |  |  |  |

Exercise 2 of 4 Recenter the map over the PERKS intersection. (Place Correct and Incorrect text feedback here when appropriate) Recenter PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 7: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Wind grid with altitude set at 30,000 feet. Range at 25. Use the centering used after the Try It for Frame 30. |
| --- | --- |
| Step 1: Left-click on Recenter | If done correctly, change cursor to an X. Change button background to gray. |
| If middle or right click or left click anywhere but Recenter: |  |
| No. You need to left-click on the Recenter button. Try again. | Highlight Recenter button. |
| Step 2: Left-click on the + PERKS area. |  |
| Correct. | Change + PERKS to the center of the display. Show Correct feedback. |
| If left click anywhere but + PERKS symbol: |  |
| No. For this exercise, please select the PERKS intersection as the new map center point. Try again. | Incorrect feedback. Highlight + PERKS. |
| If middle or right click anywhere: |  |
| For this exercise, you need to left-click on the new map center point. Try again. | Incorrect feedback. |

Branching:

| Next | 100 | Back | 80 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 100 | Exercise 3 |
| Lesson 2 |  | Wind Grid Display |  |  |  |

Exercise 3 of 4 Decrease the Wind Grid’s range one increment. (Place Correct and Incorrect text feedback here when appropriate) Range PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 7: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Wind grid with altitude set at 25,000 feet. Range at 25. Use the centering used after the Try It for Frame 30. |
| --- | --- |
| If left-click on Range: |  |
| Correct. | Change display range and scale to 20. Show Correct feedback. |
| If left click anywhere but Range: |  |
| Incorrect. You need to left click on the Range button. Try again. | Incorrect feedback. Highlight Range. |
| If middle or right click anywhere: |  |
| Left-click on Range to decrease the range. Try again. | Incorrect feedback. |

Branching:

| Next | 110 | Back | 90 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 110 | Exercise 4 |
| Lesson 2 |  | Wind Grid Display |  |  |  |

Exercise 4 of 4 Change the display’s altitude to 10,000 feet. (Place Correct and Incorrect text feedback here when appropriate) Altitude PERKS EKN Wind Grid

Quit Options Next GoTo Back Section 7: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Wind grid with altitude set at 25,000 feet. Range at 25. |
| --- | --- |
| Step 1: If left-click on Altitude: |  |
| Change Altitude button background to gray, bring up Altitude Menu as shown below. |  |
| If middle or right click anywhere or left click anywhere but Altitude: |  |
| Incorrect. You need to left click on the Altitude button. Try again. | Incorrect feedback. Highlight Altitude. |
| Step 2: If left-click on 100 or type 100 and press enter: |  |
| Good. | Correct feedback. Change display to an altitude of 10,000 feet. Background highlight 100 value in menu and change value in input area to 100. |
| If middle or right click anywhere or left click anywhere but 100: |  |
| No. You need to left-click on one zero zero. Try again. | Incorrect feedback. Highlight 100. |
| If type a value other than 100: |  |
| You have not typed the correct value. Type one zero zero. Try again. | Incorrect feedback. |

Branching:

| Next | 120 | Back | 100 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 120 | Summary |
| Lesson 2 |  | Wind Grid Display |  |  |  |

## Exercise; Correct 1st Try?. - Toggle between wind and temperature data.. 2. Recenter the display.. - Decrease the range of the display.. - Change the display’s altitude.. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 7: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

|  | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 130 | Back | 110 | If click Ex 1 | 20 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 30 | If click Ex 3 | 40 | If click Ex 4 | 50 |

| Course: |  | URET Phase 2 | Section: | 8 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 130 | Summary |
| Lesson 2 |  | Wind Grid Display |  |  |  |

(CU of clouds) Topics Covered: - Purpose of the Wind Grid Display - Switch between temperature and wind data - Recentering the display - Changing the display range - Change display altitude

Quit Options Next GoTo Back Section 6: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson on the Wind Grid Display. | CU of clouds. |
| --- | --- |

Branching:

| Next | EXIT | Back | 120 |  |  |
| --- | --- | --- | --- | --- | --- |