| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 10 | Introduction |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Topics Covered: - Trajectory Modeling (TJM) - Automated Problem Detection (APD) - Current Plan Processing - Trial Plan Processing (collage of TJM, APD, and Plan Processing graphics)

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| In this lesson, we’ll look at processes that URET uses to determine current and future aircraft positions and for identifying potential conflicts. The topics covered are: Trajectory Modeling, or TJM; Automated Problem Detection, or APD; Current Plan Processing, and Trial Plan Processing. | Photo collage. (See old M5L1, page 1) Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 20 | 4 Dimensions |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Flight Plan Data (picture of flight strip) Radar Track Data (picture of radar) Site Adaptation (picture of ARTCC) Aircraft Performance (picture of aircraft) Winds and Temperatures aloft (picture depicting wx) 4 Dimensions

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| URET combines real-time flight plan data, radar track data, site adaptation, aircraft performance characteristics, and winds and temperatures aloft to construct four-dimensional flight profiles, or trajectories, for pre-departure and active flights. | At top of screen, provide 5 boxes (in sync with audio) containing graphical images representing flight plan data, radar track data, site adaptation, aircraft performance characteristics, and winds and temperatures aloft. (See old M1L1, page 2) |
| --- | --- |
| The four-dimensions are longitude, latitude, altitude, and time. | Add four dimensions of flight profile, latitude, longitude, altitude, and time in sync with the audio |
| For active flights, it also adapts itself to the observed behavior of the aircraft, dynamically adjusting predicted speeds, climb rates, and descent rates based on the performance of each individual flight as it is tracked through en route airspace. This is to maintain aircraft trajectories to get the best possible prediction of future aircraft positions. |  |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 30 | Purpose |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Use graphic from frame 20 but add future aircraft position, label, and label time dimension Purpose

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| URET uses these trajectories to continuously detect potential aircraft conflicts against other predicted trajectories up to 20 minutes into the future, and provide strategic notification to the appropriate sector. Trajectories are also the basis for the system’s trial planning capability. | Pop-up the future aircraft position and label, other aircraft predicted trajectory, and time dimension label in sync with the audio. (See old M1L1, page 3) |
| --- | --- |

Branching:

| Next | 40 | Back | 20 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 40 | Concepts |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

- When a trajectory is created or updated for a Current Plan - What the steps taken by the system are when creating a trajectory - What Conformance Bounds are assigned to trajectories - What happens when the system detects a nonconformance Concepts

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| Along with understanding why and how this process creates a 4-dimensional predicted profile of an aircraft’s future position, you need to understand: When a trajectory is created or updated for a Current Plan, What the steps taken by the system are when creating a trajectory, What Conformance Bounds are assigned to trajectories, and What happens when the system detects a nonconformance. | Build bullets in sync with the audio (See M5L1, page 6) |
| --- | --- |

Branching:

| Next | 50 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 50 | When are Trajectories Built |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

When are Trajectories Built? Trajectories are created or updated when: - a new flight plan, flight plan amendment message, or interim altitude message is received from the Host Computer System - new or updated inter-facility flight plan information is received from a neighboring URET system - a hold message is received that indicates an aircraft is entering or leaving a holding pattern. - control of the flight is determined to be within the center’s boundaries. - a reconformance occurs for the aircraft. - none of the above occur, an existing trajectory is updated every 20 minutes.

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| A trajectory created or updated for a Current Plan when: | Build bullets in sync with the audio (See M5L1, page 7) |
| --- | --- |
| a new flight plan, flight plan amendment message, or interim altitude message is received from the Host Computer System, or new or updated inter-facility flight plan information is received from a neighboring URET system. |  |
| a hold message is received that indicates an aircraft is entering or leaving a holding pattern. |  |
| control of the flight is determined to be within the center’s boundaries. |  |
| a reconformance occurs for the aircraft. Reconformance occurs when an aircraft is determined to be outside its conformance bounds. We will explain this more later. |  |
| none of the above occur, an existing trajectory is updated every 20 minutes. |  |
| Whenever you create a Trial Plan or resubmit an existing Trial Plan, the resulting trajectory is valid until it expires. |  |

Branching:

| Next | 60 | Back | 40 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 60 | Route Conversion |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Route Conversions - Trajectory modeling process converts route string in flight plan into series of coordinates - Makes adjustments to route so that it conforms with site-specific adaptation data - Adds to converted route, any applicable preferred departure and arrival routes - If additional information is available, the route string is even further adjusted

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| The first step in creating a trajectory is route conversion. | No bullets to start. Begin with just Cleared Route: etc., and plane leaving CHO dot. (See M5L1, page 8) |
| --- | --- |
| The trajectory modeling process converts | Add Bullet 1. |
| the route string contained in the flight plan | Highlight Cleared Route: CHO..DEKAY.V38.EKN..IHD.NESTO1.PIT |
| into a series of coordinates. | Add big dots and route lines one at a time, starting with CHO |
| The process makes adjustments to the route so that it conforms with site-specific adaptation data. | Add Bullet 2. |
| The process also adds to the converted route, any applicable preferred departure and arrival routes contained within the adaptation data not contained in the original route string. | Add Bullet 3. Highlight NESTO1 PAR and arrow. |
| If additional information such as current track position or a pilot reported position is available to the trajectory modeling process, then the route string is even further adjusted; making for a more accurate route conversion. | Add Bullet 4. |

Branching:

| Next | 70 | Back | 50 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 70 | Trajectory Generation |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Trajectory Generation Planned actions are determined from one of three sources: - From altitude and speed restrictions defined in adaptation - From Host messages indicating a delay or interim altitude information - From controller input

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| Once the route is converted, the next step is to model the trajectory to include any planned actions to include altitude changes, speed changes, and delays along the route. The trajectory is also modeled to reflect the aircraft’s performance, such as climb and descent rates, and weather data. | For graphic, see M5L1, page 9. No bullets to start |
| --- | --- |
| Planned actions are determined from one or more of three sources: From altitude and speed restrictions defined in adaptation, From Host messages indicating a delay or interim altitude information, and/or From controller input. | Add bullets in sync with audio. |

Branching:

| Next | 80 | Back | 60 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 75 | Knowledge Check #1 |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Knowledge Check 1 of 1 (Place Correct and Incorrect text feedback here when appropriate) How far into the future are trajectories modeled? Click on your choice. 10 Minutes 20 Minutes 30 Minutes 40 Minutes

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	     Production Notes:

| How far into the future are trajectories modeled? Click on your choice. | Multiple choice question. Correct answer is B. Mark each incorrect answer with a red X. |
| --- | --- |
| Correct. | Show Correct! In green text, mark correct answer with check and play audio. |
| Incorrect. Trajectories are modeled further into the future. Try again. | If choose A, show Incorrect. Try again. In red text, mark answer with red X and play audio. |
| Incorrect. Trajectories are not modeled that far into the future. Try again. | If choose C or D, show Incorrect. Try again. In red text, mark answer with red X and play audio. |

Branching:

| Next | 80 | Back | 70 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 80 | Conformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Conformance - Conformance - continuously compares the radar track data with the modeled trajectory - System applies conformance bounds to the lateral, longitudinal, and vertical dimensions - Conformance bounds are adjusted to provide expanded protection during a heading change or altitude transition.

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| To make sure the modeled trajectory of a Current Plan remains accurate, the system relies on another process known as Conformance to continuously compare the radar track data with the modeled trajectory. | For graphic, see M5L1, page 10. Add bullet 1. |
| --- | --- |
| In order to do this, the system applies conformance bounds to the lateral, longitudinal, and vertical dimensions within the modeled trajectories separation boundaries. | Add bullet 2. |
| Conformance bounds, in the vertical and lateral dimensions, are adjusted to provide expanded protection during a heading change or altitude transition. | Add bullet 3. |

Branching:

| Next | 90 | Back | 70 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 90 | Lateral Conformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Lateral Conformance - 2.5 nautical miles either side of the trajectory centerline - Meets the 5 nautical mile lateral separation standard between two aircraft - Boundary may be expanded 1 nautical mile during a lateral transition or turn

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| Lateral Conformance boundaries are set at 2.5 nautical miles either side of the trajectory centerline. | For graphic, see M5L1, page 11. Add bullet 1. Highlight both 2.5 NM. |
| --- | --- |
| This value is used to meet the 5 nautical mile lateral separation standard between two aircraft. | Add bullet 2. Highlight: 5 Nautical Miles… statement |
| This boundary may be expanded 1 nautical mile during a lateral transition or turn. | Add bullet 3. |

Branching:

| Next | 100 | Back | 80 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 100 | Longitudinal Conformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Longitudinal Conformance - 1.5 nautical miles in front of and behind the trajectory centerline - Boundary may be expanded 1 nautical mile when the aircraft gets in front of or falls behind this boundary as is also possible during a turn 1 NM 1 NM 3 NM Longitudinal Conformance

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| Longitudinal Conformance boundaries are set at 1.5 nautical miles in front of and behind the trajectory centerline. | For graphic, see M5L1, page 12. Add bullet 1. Highlight 3 NM Longitudinal Conformance |
| --- | --- |
| This boundary may be expanded 1 nautical mile when the aircraft gets in front of or falls behind this boundary as is also possible during a turn. | Add bullet 2. |

Branching:

| Next | 110 | Back | 90 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 110 | Vertical Conformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Vertical Conformance - 300 ft above and below trajectory centerline - 300 ft boundary may be expanded to 1300 ft during a vertical transition Conformance Boundaries 300 ft 300 ft

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| Vertical conformance boundaries are set at three hundred feet above and below the trajectory centerline. | For graphic, see M5L1, page 13. Add bullet 1. Highlight both 300 ft. |
| --- | --- |
| This three hundred-foot boundary may be expanded to one thousand three hundred feet during a vertical transition. | Add bullet 2. |

Branching:

| Next | 115 | Back | 110 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 115 | Knowledge Check |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Knowledge Check 1 of 1 (Place Correct and Incorrect text feedback here when appropriate) Which of these statements describes Conformance monitoring? Click on your choice. Continuously compares radar track data with the modeled trajectory. Continuously detects potential aircraft conflicts against other predicted trajectories up to 20 minutes into the future. Converts the route string in a flight plan into a series of coordinates. Models the trajectory to reflect aircraft’s performance.

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	     Production Notes:

|  | Multiple choice question. Correct answer is A. Mark each incorrect answer with a red X. |
| --- | --- |
| Correct. | If select A, Show Correct! In green text, mark correct answers with check and play audio. |
| Incorrect. This describes trajectory modeling. Try again. | If choose B, show Incorrect. Try again. In red text, mark answer with red X and play audio. |
| Incorrect. This describes route conversion. Try again. | If choose C, show Incorrect. Try again. In blue text, mark answer with red X and play audio. |
| Incorrect. This describes trajectory generation. Try again. | If choose D, show Incorrect. Try again. In blue text, mark answer with red X and play audio. |

Branching:

| Next | 120 | Back | 110 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 120 | Reconformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Reconformance

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| When the reported position of an aircraft falls outside any of the three conformance bounds, the aircraft is determined to be non-conformant. | For graphic, see M5L1, page 14. Start with just smaller box with Current Plan Trajectory over it. Only show one plane – underneath the box. |
| --- | --- |
| Once this occurs, the system relies on yet another process known as Reconformance to remodel the trajectory so that it again conforms to the current aircraft location, speed, and direction. | Wipe on Reconformance circle and longer rectangle with label and another plane. |

Branching:

| Next | 130 | Back | 115 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 130 | Vertical Reconformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Vertical Reconformance

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| When the system detects | For graphic, see M5L1, page 15. |
| --- | --- |
| a climb or descent earlier, later, or at a different rate than that predicted by the trajectory, | Change to new graphic A. |
| a new trajectory is modeled by the system. This new trajectory is based on aircraft observed behavior. | Change to new graphic B. |

Branching:

| Next | 140 | Back | 120 |  |  |
| --- | --- | --- | --- | --- | --- |

A.

B.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 140 | Lateral Reconformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Lateral Reconformance 6 different types of lateral nonconformance situations: - Parallel Offset - Cut Corner - Direct to downstream point - Converging toward route - Diverging from route, stable track history - Returning to route at downstream position Click on each type of lateral nonconformance situation for a description.

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	    Production Notes:

| If a current plan trajectory is laterally out of conformance, then track history and flight plan route information are both used to determine the most likely location and path to model in order to rejoin the route. | For graphic, see M5L1, page 16. No bullets to start |
| --- | --- |
| The method used by the system to do this depends on the type of lateral non-conformance. |  |
| There are 6 different types of lateral nonconformance situations. Parallel Offset, Cut Corner, Direct to downstream point, Converging toward route, Diverging from route, stable track history, and Returning to route at downstream position. | Add bullets in sync with audio |
| Click on each type of lateral nonconformance situation for a description. | Add text prompt. Mark each choice with a green check after it is viewed. |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 150 | Parallel Offset |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

RETURN Click the button to Return to Types of Lateral Nonconformance. # Parallel Offset

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	       Production Notes:

| If the track history is parallel to the route and offset, |  |
| --- | --- |
| the new trajectory is modeled to return to the route at a downstream position. | Add line to graphic as in A below. |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

A.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 160 | Cut Corner |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

RETURN Click the button to Return to Types of Lateral Nonconformance. # Cut Corner

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	Production Notes:

| If the track history indicates the aircraft is diverging from the route near a turn fix, ABC, and the extrapolated vector is beyond the turn fix ABC, the aircraft is cutting a corner. |  |
| --- | --- |
| The new trajectory is modeled to return to the route at a downstream position or at a nearby fix. | Add line to graphic as in A below. |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

A.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 170 | Direct to Downstream Point |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

RETURN Click the button to Return to Types of Lateral Nonconformance. # Direct to Downstream Point

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	Production Notes:

| If the track history indicates the aircraft is diverging from the route and its extrapolated position is within a parameter distance from a downstream fix, | Point out in sync with audio |
| --- | --- |
| then the new trajectory is modeled direct to that fix. | Add line as in A below. |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

A.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 180 | Converging Towards Route |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

RETURN Click the button to Return to Types of Lateral Nonconformance. # Converging Towards Route

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:                                                                                                    Production Notes:

| If the track history is converging towards the route, |  |
| --- | --- |
| the new trajectory is modeled to converge on the route at an intersection point, or if the intersection point is near a fix, to that fix. | Change graphic as shown below |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

A.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 190 | Diverging from Route |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

RETURN Click the button to Return to Types of Lateral Nonconformance. # Diverging From Route, Stable Track History

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	 Production Notes:

| If the track history is stable but diverging from the route, |  |
| --- | --- |
| and an initial extrapolated vector will be applied to an intermediate point along the route, | Change graphic as in A below |
| the new trajectory is modeled to converge on the route at a turn. |  |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

A.

B.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 200 | Returning to Route at Downstream Position |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

RETURN Click the button to Return to Types of Lateral Nonconformance. # Returning to Route at Downstream Position

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	 Production Notes:

| If none of the other rules apply, |  |
| --- | --- |
| a default trajectory is modeled to converge on the route at a downstream position. | Change graphic as shown in A below. |

Branching:

| Next | 210 | Back | 130 | Parallel Offset | 150 |
| --- | --- | --- | --- | --- | --- |
| Cut Corner | 160 | Direct downstream | 170 | Converging | 180 |
| Diverging | 190 | Returning | 200 |  |  |

A.

| Course: |  | URET Phase 2 | Section: | 2 | TJM |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 210 | Longitudinal Reconformance |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Longitudinal Reconformance Current Position

Quit Options Next GoTo Back Section 2: Trajectory Modeling Repeat

- Audio:	Production Notes:

| If a current plan trajectory is longitudinally out of conformance, | See M5L1, page 17 for graphic. No label and arrow for Current Position yet. |
| --- | --- |
| say traveling faster than the predicted position, | Add arrow and label to point out Current Position of the aircraft |
| then track history is used to adjust the trajectory back into conformance. | Change graphic as in A below. |

Branching:

| Next | 220 | Back | 140 |  |  |
| --- | --- | --- | --- | --- | --- |

A.

| Course: |  | URET Phase 2 | Section: | 3 | APD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 220 | Automated Problem Detection |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Automated Problem Detection - When the system checks for conflicts - Separation values APD uses in determining conflicts - Inhibiting APD

Quit Options Next GoTo Back Section 3: APD Repeat

- Audio:	Production Notes:

| Once the trajectory is modeled or remodeled due to reconformance, it is applied to a system process known as Automated Problem Detection or APD. | See M5L1, page 18 for graphic. Start with Just APD circle. |
| --- | --- |
| APD probes aircraft trajectories to see if it detects any predicted conflicts for up to 20 minutes into the future for aircraft-to-aircraft conflicts and up to 40 minutes into the future for aircraft-to-airspace conflicts. With that in mind, let’s examine | Wipe on arrow and GPD graphic. |
| when the system checks for conflicts, | Add bullet 1. |
| The separation values APD uses in determining conflicts, | Add bullet 2. |
| and inhibiting APD. | Add bullet 3. |

Branching:

| Next | 230 | Back | 210 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | APD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 230 | Probing |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Probing - System checks a Current Plans trajectory for conflicts when the trajectory is created or amended, and when the flight plan is activated - System checks a Trial Plan trajectory when it is created - Current Plans and Trial Plans are checked for conflicts against all other Current Plans and against adapted airspaces - Trial Plans are not checked against other Trial Plans - When the system checks for conflicts with adapted airspaces, the system detects if a trajectory goes through an active military or Special Activity Airspaces or SAA’s.

Quit Options Next GoTo Back Section 3: APD Repeat

- Audio:	Production Notes:

| The system checks a Current Plans trajectory for conflicts when the trajectory is created or amended, and when the flight plan is activated. | Start with just APD circle. Add bullet 1. |
| --- | --- |
| The system checks a Trial Plan trajectory when it is created. | Add bullet 2. |
| Current Plans and Trial Plans are checked for conflicts against all other Current Plans in the region and against adapted airspaces in the region. | Add bullet 3. |
| Trial Plans are not checked against other Trial Plans. | Add bullet 4. |
| When the system checks for conflicts with adapted airspaces, the system detects if a trajectory goes through an active military or Special Activity Airspaces or SAA’s. | Add bullet 5. |

Branching:

| Next | 240 | Back | 220 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | APD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 240 | Separation Distances |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Separation Distances (Longitudinal Separation graphic from old CBI) Lateral Separation Longitudinal Separation ##### Vertical Separation

Quit Options Next GoTo Back Section 3: APD Repeat

- Audio:	Production Notes:

| Lateral, longitudinal, and vertical separation values are applied to modeled trajectories and are used by the APD process to determine whether a conflict exists between trajectories or between a trajectory and special activity airspace. | See M5L1, page 20 for graphic Add labels in sync with audio |
| --- | --- |
| The separation values for trajectories are calculated by system algorithms based on minimum separation standards as described in the 7110 point 65 and trajectory conformance bounds. |  |

Branching:

| Next | 250 | Back | 230 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | APD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 250 | Separation Buffers |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Separation Buffers

Quit Options Next GoTo Back Section 3: APD Repeat

- Audio:	Production Notes:

| Separation values used in checking for airspace conflicts are adapted based on the type of activity to take place in the airspace. | See M5L1, page 21 for graphic |
| --- | --- |
| Typically, a separation buffer is superimposed, like a force field, around the airspace boundary, so that the APD process can probe trajectories 40 minutes into the future to see if they violate the active airspace. If a buffer is not applied to the airspace, the actual airspace boundary is used. | Change graphic as shown below. |

Branching:

| Next | 260 | Back | 240 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | APD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 260 | APD Inhibiting |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

APD Inhibiting

Quit Options Next GoTo Back Section 3: APD Repeat

- Audio:	Production Notes:

| Around busy airports or areas where control remains short-term, it may not be advantageous to have the APD process detecting every possible conflict. | See M5L1, page 22 for graphic |
| --- | --- |
| Therefore, the system provides a capability for determining when or where APD should not operate. |  |
| This occurs when a portion of an aircraft’s trajectory is within airspace designated as an APD Inhibited Area. | Highlight APDIA |

Branching:

| Next | 270 | Back | 250 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Plan Processing |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 270 | Current Plan |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Current Plan Processing

Quit Options Next GoTo Back Section 4: Plan Processing Repeat

- Audio:	Production Notes:

| Current Plan processing begins with the Host Computer System sending flight plan or amendment data to URET processors networked between the Host and the D-Position consoles. | See M5L1, page 3 for graphic. Start with just flight plan box |
| --- | --- |
| Trajectory modeling, or TJM, takes flight plan data | Wipe on arrow, first text, and Trajectory Modeling circle |
| and creates 4-dimensional predicted profiles for each aircraft’s future position. | Wipe on arrow, 2nd text, and rectangle labeled Current Plan Trajectory and aircraft. |
| APD then checks each 4-dimensional predicted profile against other Current Plan trajectories and special activities airspace. | Wipe on 3rd text, arrow and APD circle. |
| If the system detects a separation problem or conflict, the Automated Problem Detection process generates data the system uses to depict aircraft and airspace conflicts. | Wipe on Detects, arrow and ACL. |

Branching:

| Next | 280 | Back | 260 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Plan Processing |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 280 | Trial Plan |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Trial Plan Processing

Quit Options Next GoTo Back Section 4: Plan Processing Repeat

- Audio:	Production Notes:

| Instead of Host data, as with the Current Plan, Trial Plan processing begins with proposed flight plan amendment data created by you at the D-Position. | See M5L1, page 3 for graphic. Start with just flight plan box |
| --- | --- |
| This proposed amendment data is sent to URET and processed by TJM. | Wipe on arrow, first text, and Trajectory Modeling circle |
| TJM then models the controller generated trial plan and creates a 4-dimensional predicted profile for the aircraft’s future position. | Wipe on arrow, 2nd text, and rectangle labeled Current Plan Trajectory and aircraft. |
| This proposed profile data is then processed by APD. APD then checks the Trial Plan trajectory against other Current Plan trajectories and adapted airspace. | Wipe on 3rd text, arrow and APD circle. |
| When a trial plan is sent to Host as a flight plan amendment and the system detects a separation problem or conflict, then the APD process generates data the system uses to depict “potential” aircraft conflicts and airspace conflicts. | Wipe on Detects, arrow and ACL. |

Branching:

| Next | 290 | Back | 270 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Plan Processing |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 290 | Track Processing |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Track Processing

Quit Options Next GoTo Back Section 4: Plan Processing Repeat

- Audio:	Production Notes:

| Track report data, such as CID, beacon code, time, position, and altitude, | See M5L1, page 3 for graphic. Start with just Track Report box |
| --- | --- |
| is submitted from Host every 12 seconds to Conformance Monitoring. Conformance Monitoring checks the track data | Wipe on arrow, first text, and Conformance Monitoring circle |
| against the Current Plan trajectory. | Add right arrow, Current Plan Trajectory label and box with aircraft below box. |
| If the track is out of conformance, | Wipe on down arrow, 2nd text, and circle labeled Reconformance. |
| Reconformance remodels the Current Plan | Add diagonal arrow and “Re-models” text |
| and generates a new 4-dimensional predicted profile. | Wipe on rectangle |
| This again generates data that the system uses to depict aircraft problems and airspace problems. | Wipe on 3rd text, arrow and APD circle. Wipe on Detects, arrow and ACL. |

Branching:

| Next | 300 | Back | 280 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 300 | Introduction |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

(photo of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 5: Exercises Repeat

- Audio:	    Production Notes:

|  | Photo of URET |
| --- | --- |

Branching:

| Next | 310 | Back | 290 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 310 | Exercise 1 |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Exercise 1 of 4 (Place Correct and Incorrect text feedback here when appropriate) Match the URET process to its definition. Click on a process and drag it to the proper position. TJM #### APD Conformance Reconformance (B) 4-dimensional flight profile based on flight plan data, radar track data, site adaptation, aircraft performance characteristics, and wind and temperatures. (C) Applies bounds to the lateral, longitudinal, and vertical dimensions within the modeled trajectories separation boundaries and continuously compare the radar track data with the modeled trajectory. (D) When the reported position of an aircraft falls outside any of the three bounds, the system remodels the trajectory so that it again conforms to the current aircraft location, speed, and direction. (A) Probes aircraft trajectories to see if it detects any predicted conflicts.

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Drag and drop. Correct answers are: - APD - TJM - Conformance - Reconformance |
| --- | --- |
| If place a process label in the correct box: |  |
| Correct. | Show Correct! In green text and play audio. Leave the label under its definition. |
| If place a process label incorrectly: |  |
| Return label to start position. | Show Incorrect. Try again in blue text. |

Branching:

| Next | 320 | Back | 300 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 320 | Exercise 2 |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Exercise 2 of 4 (Place Correct and Incorrect text feedback here when appropriate) How far into the future is the aircraft trajectory probed? Click on your choice. 20 minutes for aircraft to aircraft violations, 40 minutes for aircraft to airspace violations 40 minutes for aircraft to aircraft violations, 20 minutes for aircraft to airspace violations 40 minutes for aircraft to aircraft violation and aircraft to airspace violations 20 minutes for aircraft to aircraft violation and aircraft to airspace violations

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Multiple choice question. Correct answer is C. Mark each incorrect answer with a red X. |
| --- | --- |
| If click C: |  |
| That’s right. | Show Correct! In green text, mark correct answers with check and play audio. |
| Incorrect. There are two different times for trajectory probes. Try again. | If choose A or B, show Incorrect. Try again. In red text, mark answer with red X and play audio. |
| Incorrect. The times are reversed in this answer. Try again. | If choose D, show Incorrect. Try again. In blue text, mark answer with red X and play audio. |

Branching:

| Next | 330 | Back | 310 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 330 | Exercise 3 |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Exercise 3 of 4 (Place Correct and Incorrect text feedback here when appropriate) When creating a trial plan, which of these are checked against the trial plan trajectory for possible conflicts? Click on all that apply, then click DONE. Other trial plans. All adapted airspace. Current plans of all other aircraft. DONE

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Multiple choice question. Correct answers are A & B. Mark each incorrect answer with a red X. |
| --- | --- |
| If click A &B and then click DONE: |  |
| Very good. | Show Correct! In green text, mark correct answers with check and play audio. |
| Incorrect. Trial plans are not checked against other trial plans for potential conflicts. Try again. | If choose C, show Incorrect. Try again. In red text, mark answer with red X and play audio. |

Branching:

| Next | 335 | Back | 320 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 335 | Exercise 4 |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Exercise 4 of 4 (Place Correct and Incorrect text feedback here when appropriate) When does URET remodeling an aircraft trajectory? Click on all that apply, then click DONE. 20 minutes after every previous trajectory modeling. When a controller enters a route or altitude amendment for an aircraft. When an aircraft trajectory is out of conformance. All of the above.

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Multiple choice question. Correct answer is D. Mark each incorrect answer with a red X. |
| --- | --- |
| If click D: |  |
| Very good. | Show Correct! In green text, mark correct answers with check and play audio. |
| If click A, B, or C: |  |
| This is true, but there are more instances in which URET remodels a trajectory. Try again. | If choose A, B, or C, show Incorrect. Try again. In red text, mark answer with red X and play audio. |

Branching:

| Next | 340 | Back | 330 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 340 | Summary |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

## Exercise; Correct 1st Try?. - Match the URET process to its definition.. 2. How far into the future is the aircraft trajectory probed?. - When creating a trial plan, which of these are checked against the trial plan trajectory for possible conflicts?. - When does URET remodeling an aircraft trajectory?. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

| You have completed the end of lesson exercises. This chart indicates which exercises you completed correctly on the first try. If you wish to review any of the topics, click on a question. | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 350 | Back | 330 | If click Ex 1 | 20 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 220 | If click Ex 3 | 230 | If click Ex 4 | 50 |

| Course: |  | URET Phase 2 | Section: | 6 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 350 | Summary |
| Lesson 7 |  | APD and Plan Processing |  |  |  |

Topics Covered: - TJM - APD - Current Plan Processing - Trial Plan Processing (collage of TJM, APD, and Plan Processing graphics)

Quit Options Next GoTo Back Section 6: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson on APD and Plan processing. | Photo collage. |
| --- | --- |

Branching:

| Next | EXIT | Back | 340 |  |  |
| --- | --- | --- | --- | --- | --- |