| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 10 | Introduction |
| Lesson 1 |  | Outages |  |  |  |

Topics Covered: - Outage indications and implications for: - System Status display - Flight Plan Manager - Conflict Probe Processor - Trial Plan Manager - Host-URET connection, - Host failure - Weather data - Hardware failure - Total power outages (collage of outage graphics)

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| As in any automated system, outages can occur. You need to be able to identify and respond to these outages. In this lesson, we’ll show you examples of URET system outages and automated outage notification. The topics covered include: outage indications and implication for the System Status display, Flight Plan Manager, Conflict Probe Processor, Trial Plan Manager. We will also learn about the failure of the Host-URET connection, Host failure, Weather data, Hardware failure, and critical power outages. | Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | System Status Display |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 20 | System Status Display |
| Lesson 1 |  | Outages |  |  |  |

- Green – operational - Yellow – some degradation - Red - failed System Status Display: System Status Display Intrafacility Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 2: System Status Display Repeat

- Audio:	    Production Notes:

| You can access the System Status Display from the Status button on the tool bar. If the system status has changed, the button text turns yellow. | Tool bar. Animate button turning yellow. Animate arrow moving to Status. |
| --- | --- |
| The System Status Display shows the current system status of both intrafacility and interfacility items. The status is indicated in a box next to the function. | System status display. Make all boxes green except: Host/CP Connection is Red. |
| A green box indicates that the function is operational, | Highlight Flight Planning and its box. |
| And Red means the function is down or has failed | Highlight Host/CP Connection and its box. |
| Be aware that you may click on the yellow Status button, and the system will show no outages. This means the system has already recovered from the failure. |  |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | System Status Display |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 30 | Host/CP Connection |
| Lesson 1 |  | Outages |  |  |  |

- Indicates either Host failure or Host/CP Connection failure Host/CP Connection: System Status Display Intrafacility Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 2: System Status Display Repeat

- Audio:	    Production Notes:

| The Host/Conflict Probe connection indicates either a Host failure or failure of the Host/Conflict Probe connection. | System status display. Make all boxes green except: Host/CP Connection is Red. Highlight Host/Conflict Probe Connection. |
| --- | --- |

Branching:

| Next | 40 | Back | 20 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | System Status Display |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 40 | Weather Indicator |
| Lesson 1 |  | Outages |  |  |  |

- Includes date and time of last hourly weather report received from NWS Weather Indicator: System Status Display Intrafacility Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 2: System Status Display Repeat

- Audio:	    Production Notes:

| The weather indicator includes the date and time of the last hourly weather report received from the National Weather Service. | System status display. Make all boxes green except: Host/CP Connection is Red Weather is green. Highlight Date and Time |
| --- | --- |

Branching:

| Next | 50 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | System Status Display |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 50 | Interfacility Indicators |
| Lesson 1 |  | Outages |  |  |  |

- Green – operational - Red - failed Interfacility Indicators: System Status Display Intrafacility Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400 1425

Quit Options Next GoTo Back Section 2: System Status Display Repeat

- Audio:	    Production Notes:

| URET outages at neighboring facilities are indicated in the Interfacility portion of the System Status display. | Highlight Interfacility column. The box next I is red, the rest are green. |
| --- | --- |
| Each facility is represented by a one letter identifier. | Highlight ICAM column. |
| If an indicator is red, the time the facility’s URET connection failed will be displayed. | Highlight I (box) 1425 |

Branching:

| Next | 80 | Back | 40 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | Flight Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 80 | What You See |
| Lesson 1 |  | Outages |  |  |  |

CP Outage SHORT Conflict Data Deletion 4:14 Conflict Data Stale - RYA, conflict data and flight IDs will change to blue backgrounds with gray text - CP Outage Window appears FPM Outage – What You See: (ACL) (tool bar) Status

Quit Options Next GoTo Back Section 3: Flight Plan Manager Outage Repeat

- Audio:	    Production Notes:

| A flight plan manager outage can result from hardware, software, or network problems and prevents URET from processing flight plans. | Standard ACL and toolbar. |
| --- | --- |
| When this type of outage occurs, the alert box labels, conflict data and flight IDs in the ACL, Departure List, GPD and Plans Display will change to blue backgrounds with gray text. This indicates that the data is unreliable, or “stale.” | Change RYA, Alert boxes and flight IDs to blue background with gray text. Add bullet 1. |
| The CP outage window pops to the top of the display. | Add and highlight CP Outage Window. Add bullet 2. |
| This window indicates that the conflict data is stale, | Highlight Conflict Data Stale text. |
| And includes a countdown timer to show when the conflict data will be deleted. | Highlight Conflict Data Deletion 4:14. |
| If the outage is expected to last less than four minutes and fifteen seconds, SHORT will appear in the CP outage window. | Highlight SHORT. |
| If the outage will be longer that 4 minutes fifteen seconds, LONG will appear. | Keep highlight, change SHORT to LONG. |
| Once the countdown clock has expired, the countdown timer is removed and “conflict data stale” changes to “conflict data unavailable.” | Change the CP Outage box as shown below. |
| In addition, the conflict data will be removed from the displays. | Remove the numbers and pluses in the alert boxes. |
| Click on Status to see the changes in the System Status display for this outage. |  |
| If click any where else but Status: |  |
| Please click on Status in the tool bar. | Highlight Status in Tool Bar. |

Branching:

| Next | inactive | Back | 50 | Status | 90 |
| --- | --- | --- | --- | --- | --- |

CP Outage

LONG

Conflict Data

Unavailable

| Course: |  | URET Phase 2 | Section: | 3 | Flight Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 90 | System Status Display |
| Lesson 1 |  | Outages |  |  |  |

- RYA, conflict data and flight IDs will change to blue backgrounds with gray text - CP Outage Window appears - System Status Display shows everything unavailable except weather FPM Outage – What You See: (ACL) (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400 CP Outage LONG Conflict Data Unavailable

Quit Options Next GoTo Back Section 3: Flight Plan Manager Outage Repeat

- Audio:	    Production Notes:

| The System Status Display shows all functions unavailable, except for weather data. | Add the System Status Display to the previous screen. All the Interfacility boxes are green, as is Weather. Flight Planning, Probe, Trial Planning, and Host/CP Connection are red. Add bullet 3. |
| --- | --- |

Branching:

| Next | 100 | Back | 80 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | Flight Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 100 | What Works |
| Lesson 1 |  | Outages |  |  |  |

CP Outage SHORT Conflict Data Deletion 4:14 Conflict Data Stale - D-CRD - Flight entries displayed in URET - Weather updated - Amendments can be initiated in URET, but not displayed FPM Outage - What Works: (ACL) (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 3: Flight Plan Manager Outage Repeat

- Audio:	    Production Notes:

| Some functions at your position will still work, even when there is a flight plan manager outage. | Toolbar, Outage window, ACL, and System Status Display |
| --- | --- |
| The D-Crud operates normally, | Add bullet 1. |
| Flight entries will continue to be displayed in URET, | Add bullet 2. |
| The weather will continue to be updated, | Add bullet 3. |
| Host will still accept amendments initiated through URET, but they will not be displayed in URET. You must confirm the amendments by performing a flight plan readout in the crud. | Add bullet 4. |

Branching:

| Next | 110 | Back | 90 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | Flight Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 110 | What Doesn’t Work |
| Lesson 1 |  | Outages |  |  |  |

CP Outage SHORT Conflict Data Deletion 4:14 Conflict Data Stale - Flight entries not updated - New entries not displayed - No IFA connection - No Trial Planning FPM Outage – What Doesn’t Work: (ACL) (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 3: Flight Plan Manager Outage Repeat

- Audio:	    Production Notes:

| In addition to conflict data and current flight data becoming stale during this outage, you will loose the following functions: | Toolbar, Outage window, ACL, and System Status Display |
| --- | --- |
| Flight entries will not be updated, | Add bullet 1. |
| New entries will not be displayed in most cases, | Add bullet 2. |
| the IFA connection is not available, | Add bullet 3. |
| And trial planning is not available. This means that the trial planning menu options will be disabled, for example, in the Altitude menu. | Add bullet 4. |

Branching:

| Next | 120 | Back | 100 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | Flight Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 120 | Recovery |
| Lesson 1 |  | Outages |  |  |  |

- ACL and DL frozen. No functions available - Stale data replaced by new data FPM Outage – Recovery: (ACL) (tool bar) Status

Quit Options Next GoTo Back Section 3: Flight Plan Manager Outage Repeat

- Audio:	    Production Notes:

| When URET recovers from a Flight Plan Manager outage, the Aircraft List and the Departure list are frozen. The function buttons and all text are dimmed, indicating that none of the functions are available. | Toolbar, ACL, DL. All text and buttons are grayed out. Add bullet 1. |
| --- | --- |
| The stale data then disappears and the displays are repopulated with new information. | Quickly remove all blue and grayed out data and replace with normal data. Add bullet 2. |

Branching:

| Next | 130 | Back | 110 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Conflict Probe Processor Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 130 | What You See |
| Lesson 1 |  | Outages |  |  |  |

- RYA, conflict data and trial plan IDs will change to blue background with gray text - CP Outage Window appears CP Outage – What You See: (ACL) (tool bar) Status CP Outage Conflict Data Deletion 5:00 Conflict Data Stale (Plans Display)

Quit Options Next GoTo Back Section 4: Conflict Probe Processor Outage Repeat

- Audio:	    Production Notes:

| A conflict Probe Processor outage indicates a failure in the software that analyzes trajectories and determines whether conflicts exist. | ACL, Plans Display, and toolbar. |
| --- | --- |
| With this type of outage, alert box labels and conflict alert data will be displayed with blue shading and gray text. Flight data will not be coded as stale since flight plan processing is still operational. | On the ACL - Change Alert boxes only to blue background with gray text. On the Plans Display, change the Trial Plan IDs and alert info. Add bullet 1. |
| Since flight plan processing is still taking place, there will be no need to switch to paper strips. Therefore, the CP Outage window appears, but does not include a SHORT or LONG indicator. | Add and highlight CP Outage Window. Add bullet 2. |
| Conflict data will disappear when the countdown timer expires. | Change CP outage window as shown below and remove numbers and pluses from alert boxes. Remove alert info from Plans display. |
| Click on Status to see the changes in the System Status display for this outage. |  |
| If click any where else but Status: |  |
| Please click on Status in the tool bar. | Highlight Status in Tool Bar. |

Branching:

| Next | inactive | Back | 120 | Status | 140 |
| --- | --- | --- | --- | --- | --- |

CP Outage

Conflict Data

Stale

| Course: |  | URET Phase 2 | Section: | 4 | Conflict Probe Processor Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 140 | System Status Display |
| Lesson 1 |  | Outages |  |  |  |

- Conflict data and flight IDs will change to blue backgrounds with gray text - CP Outage Window appears - System Status Display shows Probe and Trial Planning unavailable CP Outage – What You See: (ACL) (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400 CP Outage LONG Conflict Data Unavailable

Quit Options Next GoTo Back Section 4: Conflict Probe Processor Outage Repeat

- Audio:	    Production Notes:

| The System Status Display shows Conflict Probe and Trial Planning failures. | Add the System Status Display to the previous screen. All the Interfacility boxes are green, Weather, Flight Planning, and Host/CP Connection too. Probe and Trial Planning are red. Add bullet 3. |
| --- | --- |

Branching:

| Next | 150 | Back | 130 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Conflict Probe Processor Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 150 | What Works |
| Lesson 1 |  | Outages |  |  |  |

CP Outage SHORT Conflict Data Deletion 4:14 Conflict Data Stale - D-CRD - Flight entries displayed and updated in URET - Weather updated CP Outage – What Works: (ACL) (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 4: Conflict Probe Processor Outage Repeat

- Audio:	    Production Notes:

| Most functions continue to operate under a Conflict Probe Processor outage. | Toolbar, Outage window, ACL, and System Status Display |
| --- | --- |
| The D-Crud operates normally, | Add bullet 1. |
| Flight entries will continue to be displayed and updated in URET, | Add bullet 2. |
| And the weather will continue to be updated. | Add bullet 3. |

Branching:

| Next | 160 | Back | 140 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Conflict Probe Processor Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 160 | What Doesn’t Work |
| Lesson 1 |  | Outages |  |  |  |

CP Outage SHORT Conflict Data Deletion 4:14 Conflict Data Stale - New conflict data not displayed - No Trial Planning - AMEND becomes default in Route and Altitude menus CP Outage – What Doesn’t Work: (ACL) (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 4: Conflict Probe Processor Outage Repeat

- Audio:	    Production Notes:

| In addition to conflict data becoming stale during this outage, you will loose the following functions: | Toolbar, Outage window, ACL, and System Status Display |
| --- | --- |
| New conflict information will not be displayed | Add bullet 1. |
| And Trial planning is not available, so trial plan menu options are disabled. | Add bullet 2. |
| Be aware that the Amend button becomes the default in the route and altitude menus. In other words, any selection from these menus will be sent to Host as amendments. | Add bullet 3. |

Branching:

| Next | 170 | Back | 150 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Conflict Probe Processor Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 170 | Recovery |
| Lesson 1 |  | Outages |  |  |  |

- All conflict data and trial plan data is removed - Stale data replaced by new data as received. CP Outage – Recovery: (ACL) (tool bar) Status (Plans Display)

Quit Options Next GoTo Back Section 4: Conflict Probe Processor Outage Repeat

- Audio:	    Production Notes:

| When URET recovers from a Conflict Probe Processor outage, conflict data and trial plan data are removed | Toolbar, ACL, Plans. Remove blue shading, conflict data, and trial plan data. Add bullet 1. |
| --- | --- |
| and repopulated as new information is received. | Put the information back in the normal condition, one entry at a time. Add bullet 2. |

Branching:

| Next | 180 | Back | 160 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | Trial Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 180 | What You See |
| Lesson 1 |  | Outages |  |  |  |

- Trial Plan IDs will change to blue backgrounds with gray text - No CP Outage window ### TP Manager – What You See: (tool bar) Status Plans Display 511 PDT3237 150 260 CHO..DEKAY.V36.EKN..IHD,NESTO1.PIT BTA3053 2120Z BLR555 2120Z BTA3053.T1 160 AM 501 ALT 160 NEW 1:50

Quit Options Next GoTo Back Section 5: Trial Plan Manager Outage Repeat

- Audio:	    Production Notes:

| A Trial Plan outage occurs when the software used to generate trial plans is not functioning properly. | Standard Plans Display and toolbar. |
| --- | --- |
| Only trial plan IDs in the Plans display and the GPD will change to blue backgrounds with gray text. | Change trial plan only to blue background with gray text. Add bullet 1. |
| No CP outage window appears for this type of outage. | Add bullet 2. |
| Click on Status to see the changes in the System Status display for this outage. |  |
| If click any where else but Status: |  |
| Please click on Status in the tool bar. | Highlight Status in Tool Bar. |

Branching:

| Next | inactive | Back | 170 | Status | 190 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | Trial Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 190 | System Status Display |
| Lesson 1 |  | Outages |  |  |  |

Plans Display 511 PDT3237 150 260 CHO..DEKAY.V36.EKN..IHD,NESTO1.PIT BTA3053 2120Z BLR555 2120Z BTA3053.T1 160 AM 501 ALT 160 NEW 1:50 - Trial Plan IDs will change to blue backgrounds with gray text - No CP Outage window - System Status Display shows Trial Planning unavailable TP Manager – What You See: (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 5: Trial Plan Manager Outage Repeat

- Audio:	    Production Notes:

| The System Status Display shows only a Trial Planning failure. | Add the System Status Display to the previous screen. All the Interfacility boxes are green, Weather, Flight Planning, Probe, and Host/CP Connection too. Trial Planning is red. Add bullet 3. |
| --- | --- |

Branching:

| Next | 200 | Back | 180 |  |  |
| --- | --- | --- | --- | --- | --- |

Plans Display

511  PDT3237                150  260     CHO..DEKAY.V36.EKN..IHD,NESTO1.PIT

BTA3053                2120Z

BLR555                  2120Z

BTA3053.T1                    160

AM 501  ALT  160

NEW

1:50

| Course: |  | URET Phase 2 | Section: | 5 | Trial Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 200 | What Works & What Doesn’t |
| Lesson 1 |  | Outages |  |  |  |

- Only Trial Planning features are not available - AMEND becomes default in Route and Altitude menus TP Manager – What Works & What Doesn’t: (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 5: Trial Plan Manager Outage Repeat

- Audio:	    Production Notes:

| In the event of a Trial Plan Manager Outage, everything operates normally except the trial planning features. | Toolbar, Plans Display, and System Status Display |
| --- | --- |
| Trial planning features will be unavailable and the Trial Plan buttons will be grayed out. | Add bullet 1. |
| Again, be aware that the Amend button becomes the default in the route and altitude menus. | Add bullet 2. |

Branching:

| Next | 210 | Back | 190 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | Trial Plan Manager Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 210 | Recovery |
| Lesson 1 |  | Outages |  |  |  |

- All trial plan data is removed - Stale data replaced by new data as received. TP Manager – Recovery: (tool bar) Status Plans Display 511 PDT3237 150 260 CHO..DEKAY.V36.EKN..IHD.NESTO1.PIT BTA3053 2120Z BLR555 2120Z NEW

Quit Options Next GoTo Back Section 5: Trial Plan Manager Outage Repeat

- Audio:	    Production Notes:

| When URET recovers from a Trial Plan Manager outage, trial plan data is removed | Toolbar, Plans. Remove blue shading, trial plan data from previous screen. Add bullet 1. |
| --- | --- |
| and repopulated as new information is received. | Put the information back in the normal condition (without the blue coding), one entry at a time. Add bullet 2. |

Branching:

| Next | 220 | Back | 200 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 6 | HOST-URET Connection Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 220 | What You See |
| Lesson 1 |  | Outages |  |  |  |

- Conflict Data and Flight IDs will change to blue backgrounds with gray text - No CP Outage window - IFA flight plans NOT coded as stale HOST-URET Connection Outage - What You See: (tool bar) Status (ACL) ….USA15(M-23) T/B733/F 230 / 5730 CLT./.J53.EKN..IHD.NESTO1.PIT N

Quit Options Next GoTo Back Section 6: HOST-URET Connection Outage Repeat

- Audio:	    Production Notes:

| If the connection between Host and URET has failed | Standard ACL and toolbar. Add the last entry as shown above |
| --- | --- |
| Conflict data and flight IDs will change to blue backgrounds with gray text. The alert box labels, however, do not change. This indicates that this is a Host failure and not a URET flight plan processing outage. | Change alerts and flight IDs to blue background with gray text. Add bullet 1. |
| No CP outage window appears for this type of outage. | Add bullet 2. |
| Notice that interfacility current plans without local Host flight plan data are not coded as stale. This is because the flight plan data from the adjacent facility continues to flow to URET. | Highlight the last entry – the one with three dots. Add bullet 3. |
| Click on Status to see the changes in the System Status display for this outage. |  |
| If click any where else but Status: |  |
| Please click on Status in the tool bar. | Highlight Status in Tool Bar. |

Branching:

| Next | inactive | Back | 210 | Status | 230 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 6 | HOST-URET Connection Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 230 | System Status Display |
| Lesson 1 |  | Outages |  |  |  |

(ACL) - Conflict Data and Flight IDs will change to blue backgrounds with gray text - No CP Outage window - System Status Display shows Host/CP Connection unavailable (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400 HOST-URET Connection Outage - What You See:

Quit Options Next GoTo Back Section 6: HOST-URET Connection Outage Repeat

- Audio:	    Production Notes:

| The System Status Display shows the Host CP connection as unavailable. | Add the System Status Display to the previous screen. All the Interfacility boxes are green, Weather, Flight Planning, Probe, and Trial Planning too. Host/CP Connection is red. Add bullet 3. |
| --- | --- |

Branching:

| Next | 240 | Back | 220 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 6 | HOST-URET Connection Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 240 | What Works & What Doesn’t |
| Lesson 1 |  | Outages |  |  |  |

- In most cases, All user functions will operate - Current flight entries will not update - No new flight entries will appear - Amendments can be initiated in URET, but not displayed HOST-URET Connection Outage - What Works & What Doesn’t: (tool bar) Status (ACL) System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 6: HOST-URET Connection Outage Repeat

- Audio:	    Production Notes:

| During most Host-URET Connection Outages, all user functions will operate. | Toolbar, ACL, and System Status Display Add bullet 1. |
| --- | --- |
| Because there is no communication with Host, current flight entries will not update, | Add bullet 2. |
| And no new flight entries will appear. | Add bullet 3. |
| Host will still accept amendments initiated through URET, but they will not be displayed in URET. You must confirm the amendments by performing a flight plan readout in the crud. | Add bullet 4. |

Branching:

| Next | 250 | Back | 230 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 6 | HOST-URET Connection Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 250 | Recovery |
| Lesson 1 |  | Outages |  |  |  |

- ACL and DL locked in Manual Posting mode - Stale data replaced by new data as received. HOST-URET Connection Outage - Recovery: (tool bar) Status (ACL)

Quit Options Next GoTo Back Section 6: HOST-URET Connection Outage Repeat

- Audio:	    Production Notes:

| When URET recovers from a Host-URET Connection outage, the Aircraft List and Departure list are locked in the Manual posting mode so that new flights posted during the recovery process will appear at the bottom of the list. | Toolbar, ACL. Posting Mode button is grayed out. Posting is Manual. Highlight Manual in header. Add bullet 1. |
| --- | --- |
| The stale coding is removed and the information is replaced as the new information is received. | Replace the blue information with the normal colors one at a time. Add bullet 2. |

Branching:

| Next | 260 | Back | 240 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | HOST Failure |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 260 | What You See |
| Lesson 1 |  | Outages |  |  |  |

- Conflict Data and Flight IDs will change to blue backgrounds with gray text - No CP Outage window HOST Failure - What You See: (tool bar) Status (ACL) (photo of MDM with X and text) System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 7: HOST Failure Repeat

- Audio:	    Production Notes:

| If there is a Host failure, the M-D-M will display a red X and text indicating that the “Radar Display is Down.” | Standard ACL and toolbar. |
| --- | --- |
| Conflict data and flight IDs will change to blue backgrounds with gray text. | Change alerts and flight IDs to blue background with gray text. Add bullet 1. |
| No CP outage window appears for this type of outage. | Add bullet 2. |
| The System Status Display shows the Host CP connection as unavailable. | Add the System Status Display. All the Interfacility boxes are green, Weather, Flight Planning, Probe, and Trial Planning too. Host/CP Connection is red. Add bullet 3. |

Branching:

| Next | 270 | Back | 250 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | HOST Failure |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 270 | What Works |
| Lesson 1 |  | Outages |  |  |  |

- Current flight entries continue to be displayed - Weather data is updated HOST Failure - What Works: (tool bar) Status (ACL) System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 7: HOST Failure Repeat

- Audio:	    Production Notes:

| If Host fails, current flight entries will continue to be displayed | Toolbar, ACL, and System Status Display Add bullet 1. |
| --- | --- |
| And weather data will be updated. | Add bullet 2. |

Branching:

| Next | 280 | Back | 260 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | HOST Failure |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 280 | What Doesn’t Work |
| Lesson 1 |  | Outages |  |  |  |

- D-CRD - Amendments using URET - Trial Planning HOST Failure - What Doesn’t Work: (tool bar) Status (ACL) System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400

Quit Options Next GoTo Back Section 7: HOST Failure Repeat

- Audio:	    Production Notes:

| Because this is a Host failure, the D-crud will not operate, | Toolbar, ACL, and System Status Display Add bullet 1. |
| --- | --- |
| You can not make amendments using URET, | Add bullet 2. |
| And the trial planning function is disabled. | Add bullet 3. |

Branching:

| Next | 290 | Back | 270 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 8 | Lack of Weather Data |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 290 | What You See |
| Lesson 1 |  | Outages |  |  |  |

(Wind Grid) - Red box, yellow date and time, and “Not Receiving” - URET has stopped receiving WARP data and weather data being used to calculate trajectories is based on a forecast that is no longer valid - No Weather Data - URET has neither current weather nor forecast information. Lack of Weather Data – What You See: (tool bar) Status System Status Display Intrafacility ###### Interfacility Flight Planning Probe Trial Planning Host/CP Connection Weather I C A M 5/5/03 1400 Not Receiving

Quit Options Next GoTo Back Section 8: Lack of Weather Data Repeat

- Audio:	    Production Notes:

| URET receives current weather data hourly. Updates include forecasts for the next five hours. | Start with just Wind Grid. |
| --- | --- |
| If the Weather box in the System Status Display is red and the text color of the date and time are yellow and “Not Receiving” appears below the time of the last Warp update, URET has run out of valid forecasted weather data. | Add System Status Display. Weather box is red. “Not Receiving” is yellow, while date and time remain white. Add bullet 1. |
| If URET does not have any weather data, “No Weather Data” is displayed to indicate that URET has neither current weather nor forecast information. Trajectories are then modeled on a standard day. This assumes zero wind, an assumed ground temperature, and an assumed temperature gradient for altitudes above ground level. | Remove date, time, and “Not Receiving” Replace with “No Weather Data” in yellow text on the line where the date and time were. Also, “No Weather Data “appears in wind grid in big yellow text –centered. (No arrows or numbers, just the map) |

Branching:

| Next | 300 | Back | 280 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 9 | Individual Sector Recycles |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 300 | What You See |
| Lesson 1 |  | Outages |  |  |  |

- URET displays disappear from screen - D-CRD remains and is operational Individual Sector Recycles – What You See: (tool bar) Status (ACL) (D-CRD) (Departure List)

Quit Options Next GoTo Back Section 9: Individual Sector Recycles Repeat

- Audio:	    Production Notes:

| At times, URET will recycle at your individual sector only. This is usually a software related failure that lasts from one to three minutes. | Toolbar, ACL, DL, D-CRD. |
| --- | --- |
| When this happens, the URET displays will disappear from your screen and you will have no URET functionality. | Remove ACL and DL. |
| But the D-Crud will remain on the screen and operational. | Highlight D-CRD. |

Branching:

| Next | 310 | Back | 290 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 10 | Resolving Hardware Failures |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 310 | Replacing Hardware |
| Lesson 1 |  | Outages |  |  |  |

## Flat Panel – To replace, strip bays may need to be removed. The process takes 30-45 minutes. Sector may need to be moved or position shut down. Replacing Hardware: (photo of flat panel) Flat Panel Monitor (photo of keyboard) Keyboard (photo of trackball) Trackball Click on each piece of equipment to see how its replacement will affect your position.

Quit Options Next GoTo Back Section 10: Resolving Hardware Failures Repeat

- Audio:	    Production Notes:

| There may be times when the URET position hardware needs to be replaced. | Photos of each piece of equipment. Photo and label should both be clickable. |
| --- | --- |
| Click on each piece of equipment to see how it’s replacement will affect your position. | Add prompt. |
| When each item is clicked, place the text shown for that item on the left of the screen. |  |

Branching:

| Next | 330 | Back | 300 |  |  |
| --- | --- | --- | --- | --- | --- |

Keyboard –

Quick replacement. Can be replaced without shutting down the sector position.

### Trackball –

Quick replacement. Can be replaced without shutting down the sector position.

| Course: |  | URET Phase 2 | Section: | 11 | Total Power Outage |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 330 | Using URET |
| Lesson 1 |  | Outages |  |  |  |

- URET is available - Battery backup lasts at least 30 minutes - Can be used for situational awareness during transition to backup systems - GPD provides a graphic representation of flight data for up to 20 minutes Using URET During a Total Power Outage: (tool bar) Status (GPD) (AL)

Quit Options Next GoTo Back Section 11: Total Power Outage Repeat

- Audio:	    Production Notes:

| If the facility experiences a total power outage, URET will be on battery backup and available | Toolbar, ACL, and GPD Add bullet 1. |
| --- | --- |
| for at least thirty minutes. | Add bullet 2. |
| The flight plan information on the ACL and GPD can be used to support situational awareness. | Add bullet 3. |
| It would be advantageous for controllers to use the GPD to facilitate the transition to backup systems, or if applicable, to ATC zero condition. | Add bullet 4. |

Branching:

| Next | 340 | Back | 310 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 13 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 340 | Introduction |
| Lesson 1 |  | Outages |  |  |  |

(photo of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 13: Exercises Repeat

- Audio:	    Production Notes:

|  | Photo of URET |
| --- | --- |

Branching:

| Next | 350 | Back | 330 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 13 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 350 | Exercise 1 |
| Lesson 1 |  | Outages |  |  |  |

Exercise 1 of 3 (Place Correct and Incorrect text feedback here when appropriate) Match the Outage type to the display appearance. (Portion of ACL depicting Flight Plan outage) (Portion of ACL depicting Conflict Probe outage) (Portion of ACL depicting Trial Plan outage) (Portion of ACL depicting Host-URET Connection outage) Trial Plan Manager Flight Plan Manager Conflict Probe Processor Host-URET Connection

Quit Options Next GoTo Back Section 13: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Drag and drop |
| --- | --- |
| If place a label correctly, leave it there and show Correct! In green text. |  |
| If place a label incorrectly, return it to the list and show Incorrect. Try again in blue text. |  |
| When place all labels correctly: |  |
| Very good. |  |

Branching:

| Next | 360 | Back | 340 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 13 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 360 | Exercise 2 |
| Lesson 1 |  | Outages |  |  |  |

Exercise 2 of 3 What functionality will you lose in the event of a Conflict Probe Processor outage? Click on all that apply, then click DONE. New conflict data is not displayed ## Trial Planning is not available Weather is not current Flight data becomes stale (Place Correct and Incorrect text feedback here when appropriate) DONE

Quit Options Next GoTo Back Section 13: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Multiple choice question. Correct answers are A & D. Mark each incorrect answer with a red X. |
| --- | --- |
| If click A &D and then click DONE: |  |
| Correct. | Show Correct! In green text, mark correct answers with check and play audio. |
| Incorrect. Only conflict data and trial plans are affected by a Conflict Probe Processor outage. Try again. | If choose B or C, show Incorrect. Try again. In blue text, mark answer with red X and play audio. |

Branching:

| Next | 370 | Back | 350 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 13 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 370 | Exercise 3 |
| Lesson 1 |  | Outages |  |  |  |

Exercise 3 of 3 Replacement of which URET component may require moving or shutting down the position? Click on your choice. ## MDM ## Trackball Flat Panel Display Keyboard (Place Correct and Incorrect text feedback here when appropriate)

Quit Options Next GoTo Back Section 13: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Multiple choice question. Correct answer is D. Mark each incorrect answer with a red X. |
| --- | --- |
| If click B: |  |
| Correct. | Show Correct! In green text, mark correct answers with check and play audio. |
| Incorrect. The M-D-M is not a URET component. Try again. | If choose A show Incorrect. Try again. In blue text, mark answer with red X and play audio. |
| Incorrect. The Keyboard and the trackball can be replaced without moving the sector or shutting down the position. Try again. | If choose C or D show Incorrect. Try again. In blue text, mark answer with red X and play audio. |

Branching:

| Next | 380 | Back | 360 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 13 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 380 | Summary |
| Lesson 1 |  | Outages |  |  |  |

## Exercise; Correct 1st Try?. - Match the URET outage type to the display.. 2. What functionality will you lose in the event of a Conflict Probe Processor outage?. - Replacement of which URET component may require moving or shutting down the position?. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 13: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

| You have completed the end of lesson exercises. This chart indicates which exercises you completed correctly on the first try. If you wish to review any of the topics, click on a question. | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 390 | Back | 370 | If click Ex 1 | 80 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 160 | If click Ex 3 | 310 |  |  |

| Course: |  | URET Phase 2 | Section: | 14 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 5 |  |  | Frame | 390 | Summary |
| Lesson 1 |  | Outages |  |  |  |

(collage of outage graphics) Topics Covered: - Outage indications and implications for: - System Status display - Flight Plan Manager - Conflict Probe Processor - Trial Plan Manager - Host-URET connection, - Host failure - Weather data - Hardware failure - Total power outages

Quit Options Next GoTo Back Section 14: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson on Outages. | Photo collage. |
| --- | --- |

Branching:

| Next | EXIT | Back | 380 |  |  |
| --- | --- | --- | --- | --- | --- |